Ethernet setup and installation  12/08/2008.

Note – this installation is only required for the CT4000 internal Ethernet module option.

Plug in the  Combometer power and the Ethernet cable

Install the Lantronix DeviceInstaller by running C:\EsdTest\Lantronix\DeviceInstallerI\setup.exe

or get latest version for Windows 7 >= ver 4.4.0.1 http://ltxfaq.custhelp.com/app/answers/detail/a_id/644

Install the Lantronics Com Port Redirector Manager by running C:\EsdTest\Lantronix\ComPortRedirectorManager\setup.exe

or get latest version for Windows 7 >= ver 4.3.0.3
    http://www.lantronix.com/ftp/cpr/Lantronix/4.3/4.3.0.3/Installers/Download_Web/setup.exe

- Click Programs, Lantronix, DeviceInstaller (see picture below)

  - Assign a fixed IP address if possible, else choose DCHP (obtain IP from network)

  - Choose the Telnet tab (or open a DOS window (Programs, Accessories, Command Prompt) and type: C:\>telnet <ipaddress of xport> 9999

    - Click connect

    - Hit Enter to get into Setup Mode. You have a short 2-second window to successfully get in. Start over if you miss it.

    - Go into 1 for Channel1.

    - Using the Enter key, advance through all the settings until you reach the line for 'DisConnMode'. Change this to 40 (four-zero). Hit enter.

    - Choose '9' to Save and Exit when you get back to the main menu to save your changes and reboot the XPort.

- Refer to Xport-Direct-900-445b_CPR_QS.pdf

- Open the Windows Firewall and add rules to allow UDP ports 30718, 43282, 43283 in and out.

- Click Programs Lantronix, CPR 4.1, CPR Manager

  - Click the Search (magnifying glass) icon to see the available Ethernet devices.  You should see the Xport listed.  If not then you may need to change the Xport IP address (defaults to DCHP), so return to step 1

  - Click the cable icon to add a new port

  - Select the next available com port on the “ComPorts” window and click OK

  - Double click on the new com port

  - Double click on the Xport listed in the “Devices” pane to automatically enter the IP address.

  - Click “Server Reconnect” and “Timeout reconnect” 7 seconds, 0 reconnect limit

  - Click “TCP KeepAlive, 15000 KeepAlive Time msec, 1000 interval

  - Click the “RFC 2217” check box and select “Do not tie DTR to any other signal”. And “Device Server Transmit Buffer Empty”

  - Click the Save (floppy disk) icon

  - Click the Com x Tests

  - Click Open to test it.  If it stays open, then it is OK, so close it and exit

- Check the com port settings:

  - Open the Microsoft hardware device manager (start, settings, control panel, system, hardware, device manager)

  - Click on Ports

  - Doubleclick on “Lantronix CPR port”

  - Click on “Port Settings”

  - BitsPerSecond= 9600 baud, DatBits = 8, Parity = None, Stopbits = 1, Flow control = None.

  - OK

  - If these properties do not take effect, you might have to reboot the computer.

MAC address 00204A9736E2

Software version V6.5.0.4 (070531)

Press Enter for Setup Mode

*** basic parameters

Hardware: Ethernet TPI

IP addr - 0.0.4.0/DHCP/AutoIP, no gateway set

DHCP device name : not set

*** Security

Telnet Setup is      enabled

TFTP Download is     enabled

Port 77FEh is        enabled

ECHO is              disabled

Enhanced Password is disabled

Port 77F0h is        enabled

*** Channel 1

Baudrate 9600, I/F Mode 4C, Flow 00

Port 10001

Connect Mode : C0

Send '+++' in Modem Mode enabled

Show IP addr after 'RING' enabled

Auto increment source port disabled

Remote IP Adr: --- none ---,

Port 00000

Disconn Mode : 40 (Telnet Com Port Cntrl Enabled)

Flush   Mode : 00

Terminal name:

*** Expert

TCP Keepalive    : 45s

ARP cache timeout: 600s

Monitor Mode @ bootup : enabled

MTU Size: 1400

Alternate MAC: disabled

Ethernet connection type: auto-negotiate

Change Setup:

# 0 Server

1 Channel 1

5 Expert

6 Security

7 Defaults

8 Exit without save

9 Save and exit            Your choice ? 0

IP Address : (000) .(000) .(004) .(000)

Set Gateway IP Address (N) ?

Netmask: Number of Bits for Host Part (0=default) (0)

Change telnet config password (N) ?

Change DHCP device name (not set) ? (N) ?

Change Setup:

0 Server

# 1 Channel 1

5 Expert

6 Security

7 Defaults

8 Exit without save

9 Save and exit            Your choice ? 1

Baudrate (9600) ?

I/F Mode (4C) ?

Flow (00) ?

Port No (10001) ?

ConnectMode (C0) ?

Send '+++' in Modem Mode  (Y) ?

Show IP addr after 'RING'  (Y) ?

Auto increment source port  (N) ?

Remote IP Address : (000) .(000) .(000) .(000)

Remote Port  (0) ?

DisConnMode (40) ?  (Telnet Com Port Cntrl Enabled)

FlushMode   (00) ?

DisConnTime (00:00) ?:

SendChar 1  (00) ?

SendChar 2  (00) ?

Terminal name () ?

Change Setup:

0 Server

1 Channel 1

# 5 Expert

6 Security

7 Defaults

8 Exit without save

9 Save and exit            Your choice ? 5

TCP Keepalive time in s (1s - 65s; 0s=disable):  (45) ?

ARP Cache timeout in s (1s - 600s) :  (600) ?

Disable Monitor Mode @ bootup (N) ?

MTU Size (512 - 1400):  (1400) ?

Enable alternate MAC (N) ?

Ethernet connection type:  (0) ?

Change Setup:

0 Server

1 Channel 1

5 Expert

# 6 Security

7 Defaults

8 Exit without save

9 Save and exit            Your choice ? 6

Disable Telnet Setup (N) ?

Disable TFTP Firmware Update (N) ?

Disable Port 77FEh (N) ?

Disable ECHO ports (Y) ?

Enable Enhanced Password (N) ?

Disable Port 77F0h (N) ?

Change Setup:

0 Server

1 Channel 1

5 Expert

6 Security

7 Defaults

8 Exit without save

9 Save and exit            Your choice ? 9